### Configuration remote VPN

Hostname

MediaContactGate

Adressage des interfaces

| Interface | Zone | Adresse IP | Masque de sous Réseau | Passerelle | Commentaires |
| --- | --- | --- | --- | --- | --- |
| GigabitEthernet0/0 | Outside | 196.251.154.6 | 255.255.255.224 | 196.251.154.2 | Reçois la connexion ISP |
| GigabitEthernet0/1 | Inside | 10.0.5.241 | 255.255.255.0 |  | A connecté au Réseau Local Client |

Comptes Utilisateurs

| Compte username | Password |
| --- | --- |
| XXXXXXXX | XXXXXXXX |
| XXXXXXXX | XXXXXXXX |
| XXXXXXX | XXXXXXXX |

Mode d’accès : ssh / console / asdm

Routes :

route outside 0.0.0.0 0.0.0.0 196.251.154.2

route inside 10.10.214.227 255.255.255.255 10.0.5.252

route inside 10.77.0.0 255.255.0.0 10.0.5.252

route inside 172.21.34.83 255.255.255.255 10.0.5.252

Plage d’adresse attribuée au client VPN

192.168.200.100 – 192.168.200.200 / 24

Information DNS

| IP Serveur DNS | 10.0.5.1 |
| --- | --- |
| Nom de domaine | benin.groupmediacontact.com |

Access list

SPLIT TUNELLING

access-list VPN-Split standard permit 10.0.5.0 255.255.255.0

access-list VPN-Split standard permit 10.77.0.0 255.255.0.0

access-list VPN-Split standard permit host 172.21.34.83

access-list VPN-Split standard permit host 10.10.214.227

INSIDE

access-list inside_access_in extended permit ip any any

OUTSIDE

access-list outside_access_in extended permit ip object NETWORK_OBJ_192.168.200.0_24 any

PAT (VPN to MCB)

object network NETWORK_OBJ_192.168.200.0_24

subnet 192.168.200.0 255.255.255.0

nat (outside,inside) source dynamic NETWORK_OBJ_192.168.200.0_24 interface

PAT (MCB to Internet)

object network NAT-LAN

subnet 10.0.5.0 255.255.255.0

nat (inside,outside) dynamic interface